CITY OF GRAND RAPIDS

SPECIAL PROVISION

FOR

Roadway grading

ENG/City of Grand Rapids	1 of 2	org:12/14/21

rev:xx/xx/xx

a.	Description.  This work consists of all earthwork required to construct the proposed cross sections within the full depth construction limits shown on the Plans, including the removal of subgrade material to the line and grade called for on the Plans and the preparation of the subgrade.  This work shall include removal of miscellaneous structures and materials.  All work shall be completed in accordance with section 205 of the Standard Specifications for Construction and this special provision.  This work shall also include grading of all curb ramps within the project limits of all street segments.

b.	Materials.  Additional material required to achieve the typical cross sections shall meet the requirements of Granular Material, Class II.

c.	Construction.  As required to achieve proposed cross sections, Roadway Grading includes, but is not limited to, the following work.

1.	Removing miscellaneous structures and materials.

2.	Removing trees less than 6 inches in diameter.

3.	Removing rocks or boulders less than 0.5 cubic yard in volume.

4.	Removing landscaping items within the right-of-way, including plants, edging, landscaping blocks, and other miscellaneous landscaping items.

5.	Sawing existing pavement when not called out separately in the Plans or included in the Proposal.

6.	Topsoil stripping.

7.	Grading to Heavy Line as shown on the Plans and compacting subgrade shown on typical sections.

8.	Compacting of existing subbase or aggregate base left in place during utility or road reconstruction and meeting the proposed cross-section.

9.	Final grading to plan alignment and grades.

10.	Maintaining driveway access.

11.	Maintaining a lane for emergency and local traffic along and parallel to the street.

12.	Protecting and maintaining existing gates and fences.

13.	Protecting and maintaining existing retaining walls.

14.	Temporarily holding and protecting utility and light poles as needed for construction of roadway and underground utilities.

15.	Items identified in the Plans as included in Roadway Grading.

d.	Measurement and Payment.  The completed work, as described, will be measured and paid for at the contract unit price using the following pay items:

Pay Item	Pay Unit

Roadway Grading 	Station

Roadway Grading will be measured along the proposed centerline alignment in stations.  Roadway Grading includes work on both sides of the roadway and at driveways and intersections to the ground disturbance and paving limits shown on the plans.  Pavement, curb and gutter, and sidewalk removal and other items specifically excluded in this Special Provision will be paid for separately.

1.	In the event the existing Subbase material located between the Heavy Line and Aggregate Base, as shown on the Plans, meets the requirements of Class II sand, and grading to the Heavy Line is not completed, the Subbase shall not be measured and paid separately and is included in Roadway Grading.  No deduction of payment for Roadway Grading will be made or credit will be given by the Contractor to the City for not grading to the Heavy Line in these areas.

2.	When grading to the Heavy Line is completed by the Contractor, Subbase, CIP shall be measured and paid for both existing material meeting Class II requirements utilized from onsite as well as Class II material furnished from off site.

Estimated earthwork quantities for Roadway Grading are listed below and are for information only:

- Excavation, Earth = _____ cyd (Inclusive of topsoil stripping)

- Embankment (Embankment, CIP (CI II)) = _____ cyd