city of grand rapids

special provision

for

MAINTAINING TRAFFIC

ENG/City of Grand Rapids	1 of 4	org:09/14/21

rev:XX/XX/XX

- Description. This work shall consist of the necessary provisions for Maintaining Traffic in and around the Work, as shown on the Plans, listed in the Proposal, and specified in this Special Provision.  All work in this Special Provision shall be in accordance with section 812 of the Standard Specifications for Construction, except as modified herein.

- Materials.

  - General.  During non-working periods, any work site with uncompleted work shall have advance signs and plastic drums, at specific locations, as directed by the Engineer.

  - Temporary Signs.

    - Advanced signing for work zone sequences shall be as shown in attached Maintaining Traffic Typical(s).

    - Minimum signing for Detours shall be as shown on attached Typical(s). Additional signing may be required to comply with the Michigan Manual of Uniform Traffic Control Devices and as required by Mobile GR. All detour signing shall be in accordance with the approved traffic control plan.

    - Distances between construction warning, regulatory, and guide signs shown on the Typical(s) are approximate and may require field adjustment, in accordance with the MMUTCD, or as directed by the Engineer.

    - No detour signing shall be erected until the detour and the road closures are in effect. All detour signing shall be removed immediately upon the road closure being open to traffic. Sign covers for detour signing shall not be paid for separately but are included in payment for Minor Traf Devices.

  - Channelizing Devices.

    - Channelizing devices required shall be plastic drums with fluorescent sheeting.

    - (XX) Plastic drums | Channelizing Devices are included, besides the drums shown on Typical(s), to be used at the discretion of the Engineer.

    - (X) Type III Barricades are included, besides the barricades shown on Typical(s), in the quantities to be used at the discretion of the Engineer.

- Construction.

  - General.

    - The Contractor shall submit a proposed construction sequencing plan to the Engineer at the preconstruction meeting for review and approval. The Contractor shall schedule and sequence construction of the Work to meet the criteria outlined in this Special Provision.

  - Construction Influence Area (CIA).

    - The CIA shall include the right-of-way of the following roadways, within the approximate limits described below:

      - XXXX Avenue from XXXX Street to XXXX Street.

      - XXXX Street from XXXX Avenue to XXXX Avenue.

  - Construction Sequences.  The traffic control required by this Special Provision for work in the Project area is based on the suggested sequence of operations described below (if included).  An alternate traffic control plan may be used by the Contractor, subject to review and approval by the Engineer. Whether using the suggested plan or proposing an alternate plan, the Contractor shall submit a detailed traffic control plan to the Engineer for review prior to implementing any traffic restrictions.

    - Phase 1: Limits include….  In addition to the traffic restrictions outlined in this Special Provision, the following traffic restrictions shall also apply during construction:

    - Phase 2: Limits include….  In addition to the traffic restrictions outlined in this Special Provision, the following traffic restrictions shall also apply during construction:

  - Maintaining Traffic.

    - Restrictions.  At no time shall any two adjacent intersections be closed to traffic concurrently.

      - General.

        - No work shall be performed during the following 20XX holiday periods as defined by the Engineer.

          - Memorial Day – Day of Week, Month Day, Year at 3PM thru Day of Week, Month Day, Year at 7 AM

          - July 4th, Day of Week, Month Day, Year at 3 PM thru Day of Week, Month Day, Year at 7 AM

          - Labor Day – Day of Week, Month Day, Year at 3PM thru Day of Week, Month Day, Year at 7 AM

          - ADD ADDITIONAL SPECIAL EVENTS IMPACTING THE PROJECT

        - All work shall be conducted during day time hours only.  Night work may be permitted, at the discretion of the Engineer.  However, any additional cost for maintaining traffic, including necessary lighting, shall be borne by the Contractor.

        - When a lane or part of a lane is closed the speed limit shall be set at 25 miles per hour and the R2-1's shown on the attached figures shall reflect this speed.

        - Vehicular and pedestrian access to all business and parking areas shall be maintained at all times.

        - Keep all side streets and alleys free of construction related materials, equipment, and activities to allow clear visibility and access to all properties and businesses.

        - Access for construction vehicles between the travel lanes and work areas will be restricted to specific locations.  The number of access points and their locations will require the prior approval of the Engineer.

        - Undercuts or excavations immediately adjacent to active traffic lanes shall be restored to no less than a 1 on 4 slope at the end of each working day unless otherwise approved by the Engineer.  Fencing shall be required to protect open trenches during non-working hours and shall be provided by the Contractor as part of the trenching item utilized.

        - Once work begins within a phase, the work shall be aggressively pursued in a continuous manner until all work is completed. Street reconstruction and restoration shall closely follow the completion of underground construction, including curb and gutter, drive approaches, sidewalk, bituminous wearing course, and parkway restoration. Underground work may proceed to the next phase only after the Contractor has satisfactorily demonstrated to the Engineer that completion of restoration is closely following the underground work.

        - Street name signs shall be maintained within the construction area for the duration of the project.  Street name signs shall be visible from all intersection corners.  If the existing street name sign and support are to be removed, and there is no alternate support available, the Contractor shall erect a separate post (provided by the City).  The Contractor shall pick up the post and any required hardware from the City Sign Shop (509 Wealthy Street, SW, Phone: 616-456-3279).  The cost for this work shall be included in Minor Traf Devices.

        - Except when detoured, a sidewalk (temporary or permanent) shall be maintained on one side of street or the other at all times for each street within the project area.

        - Ingress and egress to all buildings for pedestrians and vehicles shall be maintained. Temporary driveways and roads may be constructed.

        - Contractors, employees, and subcontractors shall not park vehicles or store equipment in areas that are needed for residential or commercial parking.

        - Removing, obstructing, relocating, or otherwise tampering with a regulatory traffic sign without prior approval of the Engineer is prohibited.

      - Project Specific Restrictions

      - Project Specific Phasing Restrictions

- Measurement and Payment.  	The estimate of quantities for maintaining traffic is based on signing and related traffic control devices for the detours and traffic restrictions shown in the Maintaining Traffic Figure(s), Typical(s) or Plans.

  - Any additional signing or maintaining traffic devices required to expedite the construction shall be at the Contractor's expense.